# ── 设备检查 ──

busctl --user introspect bmc.kepler.hwproxy /bmc/kepler/Chip/SPIFlash/SPIFlash_1_01

# ── FlashIO 接口 ──

# Write: 写入 8 字节到偏移 256

busctl --user call bmc.kepler.hwproxy /bmc/kepler/Chip/SPIFlash/SPIFlash_1_01 bmc.kepler.Chip.FlashIO Write "a{ss}uay" 0 256 8 222 173 190 239 1 2 3 4

# Read: 从偏移 256 读取 8 字节

busctl --user call bmc.kepler.hwproxy /bmc/kepler/Chip/SPIFlash/SPIFlash_1_01 bmc.kepler.Chip.FlashIO Read "a{ss}uu" 0 256 8

# RawWrite: 发送 SPI Write Enable (0x06)

busctl --user call bmc.kepler.hwproxy /bmc/kepler/Chip/SPIFlash/SPIFlash_1_01 bmc.kepler.Chip.FlashIO RawWrite "a{ss}ay" 0 1 6

# RawRead: 发送 0x05 读取状态寄存器

busctl --user call bmc.kepler.hwproxy /bmc/kepler/Chip/SPIFlash/SPIFlash_1_01 bmc.kepler.Chip.FlashIO RawRead "a{ss}uay" 0 1 1 5

# ── BlockIO 接口 ──

# Write

busctl --user call bmc.kepler.hwproxy /bmc/kepler/Chip/SPIFlash/SPIFlash_1_01 bmc.kepler.Chip.BlockIO Write "a{ss}uay" 0 256 8 222 173 190 239 1 2 3 4

# Read

busctl --user call bmc.kepler.hwproxy /bmc/kepler/Chip/SPIFlash/SPIFlash_1_01 bmc.kepler.Chip.BlockIO Read "a{ss}uu" 0 256 8

# ── bmc.kepler.Chip 属性 ──

# HealthStatus

busctl --user get-property bmc.kepler.hwproxy /bmc/kepler/Chip/SPIFlash/SPIFlash_1_01 bmc.kepler.Chip HealthStatus

# LockStatus

busctl --user get-property bmc.kepler.hwproxy /bmc/kepler/Chip/SPIFlash/SPIFlash_1_01 bmc.kepler.Chip LockStatus

# ── SetAccessibility ──

# 禁用访问 2 秒

busctl --user call bmc.kepler.hwproxy /bmc/kepler/Chip/SPIFlash/SPIFlash_1_01 bmc.kepler.Chip SetAccessibility "a{ss}a{ss}bq" 1 "Requestor" "TestClient" 1 "Requestor" "TestClient" false 2

# 恢复访问

busctl --user call bmc.kepler.hwproxy /bmc/kepler/Chip/SPIFlash/SPIFlash_1_01 bmc.kepler.Chip SetAccessibility "a{ss}a{ss}bq" 1 "Requestor" "TestClient" 1 "Requestor" "TestClient" true 1

# ── SetLockStatus ──

# ClientA 锁定 (op_type=1, lock_time=30)

busctl --user call bmc.kepler.hwproxy /bmc/kepler/Chip/SPIFlash/SPIFlash_1_01 bmc.kepler.Chip SetLockStatus "a{ss}a{ss}yu" 1 "Requestor" "ClientA" 1 "Requestor" "ClientA" 1 30

# ClientA 续锁 (lock_time=60)

busctl --user call bmc.kepler.hwproxy /bmc/kepler/Chip/SPIFlash/SPIFlash_1_01 bmc.kepler.Chip SetLockStatus "a{ss}a{ss}yu" 1 "Requestor" "ClientA" 1 "Requestor" "ClientA" 1 60

# ClientB 解锁 (应返回 3=RequestorMismatched)

busctl --user call bmc.kepler.hwproxy /bmc/kepler/Chip/SPIFlash/SPIFlash_1_01 bmc.kepler.Chip SetLockStatus "a{ss}a{ss}yu" 1 "Requestor" "ClientB" 1 "Requestor" "ClientB" 0 0

# ClientA 解锁 (应返回 0=OK)

busctl --user call bmc.kepler.hwproxy /bmc/kepler/Chip/SPIFlash/SPIFlash_1_01 bmc.kepler.Chip SetLockStatus "a{ss}a{ss}yu" 1 "Requestor" "ClientA" 1 "Requestor" "ClientA" 0 0

# ClientA 重复解锁 (应返回 2=AlreadyUnlocked)

busctl --user call bmc.kepler.hwproxy /bmc/kepler/Chip/SPIFlash/SPIFlash_1_01 bmc.kepler.Chip SetLockStatus "a{ss}a{ss}yu" 1 "Requestor" "ClientA" 1 "Requestor" "ClientA" 0 0

# 超时锁定 lock_time=1801 (应返回 1=InvalidParameter)

busctl --user call bmc.kepler.hwproxy /bmc/kepler/Chip/SPIFlash/SPIFlash_1_01 bmc.kepler.Chip SetLockStatus "a{ss}a{ss}yu" 1 "Requestor" "ClientA" 1 "Requestor" "ClientA" 1 1801